New hire safety practices and fit for duty checklist

Before being referred to work as a UTP employee, each new hire is required to affirm that s/he understands the following information by checking the corresponding boxes and signing this form.

1. ❏Overview of the Work: Your job will consist of the unloading, assembly and disassembly of        equipment that supports traveling performances, concerts and entertainment attractions.

2. ❏ Always remember it’s during these processes that preventable injuries most often occur.

3. ❏ Fitness for Duty means that by signing this document and checking its corresponding boxes, you               are able to understand and practice this information. It also means that you will

a. ❏ Always present yourself as adequately rested, fed, and able to physically, mentally and safely cooperate with others in this line of work.

b. ❏ Always wear snug-fitting mechanic style gloves and steel toe shoes or boots (see links for purchasing these items elsewhere on this website)

c. ❏Always arrive and remain chemically free of any substance, legal or not, that is known to possibly impair your ability to safely and effectively do your job.

d.  ❏ Always be mindful that UTP employees agree to be tested for chemical substances as routine procedure prior or during any post injury medical treatment.

4.❏Always and immediately report any workplace injury, illness or hazardous condition to your steward.

a. ❏Be familiar with the information necessary to complete UTP accident report forms which are available on this website.

5. ❏Awareness of Forklifts, Scissor lifts, Snorkels and other machinery:

a. ❏ Establish eye contact and awareness with drivers working in your area.

b. ❏ Remain clear of travel lanes. Remain alert for reverse gear warning beeps and beacons.

c. ❏ Keep well away from wheels, masts, loads and forks.  Never ride as a passenger.

6.  Proper lifting techniques:

a. ❏ Mentally go over the entire lift before you commit. Center the load between your feet.

b. ❏With head back and back straight, bend your knees to lower your body to reach the load

c. ❏ Always lift with your legs—never from your waist. Keep the load centered and close.

d. ❏ Pivot the load by redirecting your feet and body, never by twisting at your back or waist.

7.  Managing Rolling Containers (RC) e.g. racks, road cases, dollies, hampers—anything on wheels:

a. ❏Never attempt to move an RC you can’t see around or safely control without help.

b. ❏Never pace your RC faster than a steady, controlled walk.

c. ❏ Never leave your RC unattended or insecurely blocked on an unlevel surface.

d. ❏Never place your hands on a RC where they can be crushed by shifting contents.

e. ❏Always maintain a distance of 6 + feet between your RC and the pusher ahead or behind.

8.  Slip/Trip/Fall Hazards:											 a. ❏ Never walk while using handheld electronics or while distracted.				 b. ❏ Practice looking up and around, and conscious of what you are doing at all times.			 c. ❏ Never step backwards without really SEEING what’s behind you first.				 d. ❏ Beware of stepping on anything between the sole of your shoe and the floor.

Applicant’s Signature__________________________ Cell phone#________________________

Printed name_______________________________ Date_________